MEADOW LAKE AIRPORT FACILITIES REPORT

12 August 2008

Ron Lee

Items to be discussed

1)  Airport lights

2)  Pavement Assessment

3)  5010 Inspection

4)  Repaint Runway Markings

5)  Compass Rose

1)  Airport Lights 
a.
54 of 60 runway lights are operational (one more than last month).

b.
15 of 16 threshold lights are operational (same as last month).

c.
189 of 198 taxiway lights are operational (five more than last month)

d.
5 of 7 runway entrance signs are operational (same as last month).

Operational capability is charted in Figure 1 below

[image: image1.wmf]Runway Light Status

0

10

20

30

40

50

60

70

80

90

100

6/10/08

6/15/08

6/20/08

6/25/08

6/30/08

7/5/08

7/10/08

7/15/08

7/20/08

7/25/08

7/30/08

8/4/08

8/9/08

Date

Percentage Operational

Runway

lights (60)

Taxiway

lights (198)

Threshold

lights (16)

Runway

entrance

signs (7)


Figure 1:  Runway Lighting System Operational Capability 
1)  Airport Lights: (continued):  

Objective:  100% threshold and runway light operational status within two months (Sep 08 board meeting).  Greater than 95% taxiway light operational status within two months (Sep 08 board meeting).

Met taxiway objective (currently 95.45% operational).  Parts to install transforners have arrived.  Initial effort to change out transformers began this morning.

2)  Pavement Assessment:  The CDoT Pavement Condition Inspection is scheduled on 14 August.

3)  5010 Inspection:  Afternoon of 14 August 2008.  Need to correct information that flows into AFD and Airnav.

4)  Repaint Runway 15 Markings:  The runway markings have degraded since the resealing in 2006.  Restoring them to like-new condition will provide an immediate positive first impression for arriving pilots and eliminate the poor visibility of the markings.  Painting is not subsidized by grants.  Considered normal maintenance by CDoT.

PROPOSAL/Motion :  Use volunteers to repaint the Rwy 15 numbers and touchdown lines as a test case.  Use Diamond Vogel UC-1501 white paint ($10.61/gallon) and Type I reflective bubbles ($45 per 50 pound bag).  Total area around 4100 sq ft.  Applied at 60 sq ft/gallon is 68 gallons ($772) plus 350 pounds of bubbles ($337).  Add painting supplies and drinking water/ice.  Estimated worst case cost is around $1300 (allows for additional paint if needed).

5)  Compass Rose:  None known to exist on airport.  Suggest painting by 99s on runway 33 runup area if that improvement is implemented.

� EMBED Unknown  ���








PAGE  
2

[image: image2.wmf]Runway Light Status

0

10

20

30

40

50

60

70

80

90

100

6/10/08

6/15/08

6/20/08

6/25/08

6/30/08

7/5/08

7/10/08

7/15/08

7/20/08

7/25/08

7/30/08

8/4/08

8/9/08

Date

Percentage Operational

Runway

lights (60)

Taxiway

lights (198)

Threshold

lights (16)

Runway

entrance

signs (7)

_1280053087.xls
Chart1

		39609		39609		39609		39609

		39610		39610		39610		39610

		39614		39614		39614		39614

		39616		39616		39616		39616

		39619		39619		39619		39619

		39624		39624		39624		39624

		39632		39632		39632		39632

		39646		39646		39646		39646

		39671		39671		39671		39671



Runway lights (60)

Taxiway lights (198)

Threshold lights (16)

Runway entrance signs (7)

Date

Percentage Operational

Runway Light Status

61.7

66.2

56.25

14.3

75

84.8

56.25

14.3

85

87.9

75

14.3

88.3

87.9

81.25

14.3

88.3

88.9

87.5

28.6

88.3

92.9

87.5

71.4

88.3

92.9

93.75

71.4

90

95.45

93.75

71.4

90

95.45

93.75

71.4



Sheet1

				Runway lights out		Taxiway lights out		Threshold lights out		Rwy entrance signs out		Runway lights (60)		Taxiway lights (198)		Threshold lights (16)		Runway entrance signs (7)

		10-Jun-08		23		66		7		6		61.7		66.2		56.25		14.3

		11-Jun-08		15		36		7		6		75		84.8		56.25		14.3

		15-Jun-08		9		24		4		6		85		87.9		75		14.3

		17-Jun-08		7		24		3		6		88.3		87.9		81.25		14.3

		20-Jun-08		7		22		2		5		88.3		88.9		87.5		28.6

		25-Jun-08		7		14		2		2		88.3		92.9		87.5		71.4

		3-Jul-08		7		14		1		2		88.3		92.9		93.75		71.4

		17-Jul-08		6		9		1		2		90		95.45		93.75		71.4

		11-Aug-08		6		9		1		2		90		95.45		93.75		71.4

		31-Aug-08

				Assumed 198 Taxiway, 60 runway and 18 REIL lights





Sheet1

		



Runway lights (60)

Taxiway lights (198)

Threshold lights (16)

Runway entrance signs (7)

Date

Percentage Operational

Runway Light Status



Sheet2

		





Sheet3

		






